National Emission Standards for Hazardous Air Pollutants for Surface Coating of

Automobiles and Light-Duty Trucks, 40 CFR 63, Subpart IIII

Terms Last Revised: 5/27/2016

Template for adhesives and sealers other than materials used in glass bonding	New Source

A.	State and Federally Enforceable Section

I.	Applicable Emissions Limitations and/or Control Requirements

1.	The average organic HAP emissions from all adhesive and sealer materials (other than materials used in glass bonding) shall not exceed:

0.010 kg organic HAP/kg of adhesive and sealer material used each month; or

0.010 pound of organic HAP/pound of adhesive and sealer material used each month.

[40 CFR 63.3090(c)] and [40 CFR 63.3091(c)]

I.	Additional Terms and Conditions

1.	The permittee shall comply with the applicable provisions of the National Emissions Standards for Hazardous Air Pollutants (NESHAP) for Surface Coating of Automobiles and Light-duty Trucks, as promulgated by the United States Environmental Protection Agency under 40 CFR Part 63, Subpart IIII.

The final rules found in 40 CFR Part 63, Subpart IIII establish national emission standards for hazardous air pollutants (HAP), work practice standards, operating limitations, and compliance requirements for automotive coating operations.  The affected source is the collection of all of the following operations for or from the surface coating of automobiles and light-duty trucks:

a.	all coating operations as defined in 40 CFR 63.3176;

b.	all storage containers and mixing vessels in which coatings, thinners and/or cleaning materials are stored or mixed;

c.	all manual and automated equipment and containers used for conveying coatings, thinners, and cleaning materials; and

d.	all storage containers and all manual and automated equipment and containers used for conveying waste materials generated by the coating operations.

The permittee shall be subject to this NESHAP upon the startup of the coating operations.  The first day of operations shall begin the initial compliance period, which shall end on the last day of the first full month of operations, i.e., the last day of the month following the month of startup, if operations do not begin on the first day of the month.

[40 CFR 63.3083], [40 CFR 63.3082(b) and(d)], [40 CFR 63.3150], and [40 CFR 63.3160(a)(3)]

2.	If every individual material used within a group of materials applied in the adhesive or sealer application meets the emission limit in 40 CFR 63.3090(c) or 63.3091 (c) (as applicable), the permittee may demonstrate compliance with such emission limit by documenting the name and the organic HAP content of each material in the group during the initial compliance period.  If any individual material within a group of materials applied, exceeds the emission limitation for that coating group, the mass average organic HAP content for the coating operations must be calculated as required in 40 CFR 3151(d) and this permit.

[40 CFR 63.3151]

III.	Monitoring and/or Record Keeping Requirements

1.	The permittee shall collect and record the following information each month for the all materials applied in the adhesive and sealer applications (excluding glass bonding) at the automotive facility:

a.	the name and identification number of each adhesive and sealer employed in the adhesive and sealer applications, including information from the supplier or manufacturer, formulation data, and/or testing data;

b.	the number of gallons or liters of each adhesive and sealer and solvent/solvent blend employed;

c.	the density of each adhesive and sealer and solvent/solvent blend employed, in pounds per gallon or kilograms per liter, using ASTM Method D1475-98 or the density supplied by the manufacturer of each material;

d.	the mass fraction of organic Hazardous Air Pollutants (HAP) for each adhesive and sealer and solvent applied during the month, as a weight fraction, i.e., pound of HAP/pound of adhesive, sealer, and solvent material, or kg/kg, using one of the following methods:

i.	Method 311 from 40 CFR Part 63, Appendix A;

ii.	Method 24 from 40 CFR Part 60, Appendix A if all nonaqueous volatile matter is to be used for the mass fraction of HAP; or

iii.	information from the supplier or manufacturer of the materials, where the mass fraction of organic HAP can be calculated from the density and the mass of HAP per gallon of each material (pound HAP/gallon of material  pounds/gallon of material, or calculated in kg/liter);

e.	the monthly mass average organic HAP content of adhesives and sealers, calculated as follows:

r					 	r

Cavg, as =  [∑ (VOLas, j) (Das, j) (Was, j)]  [∑ (VOLas, j) (Das, j)]

j =1						j =1

where:

Cavg, as is the mass average organic HAP content of adhesive and sealers used, kg/kg.

VOLas, j is the volume of adhesive or sealer “j” used, in gallons or liters.

Das, j is the density of adhesive or sealer “j” used, in pounds/gallon or kg/liter.

Was, j is the mass fraction of organic HAP in adhesive or sealer “j” used, in pound/pound or kg/kg.

r is the number of adhesives and sealers used; and

f.	if Cavg does not meet the limitation contained in 40 CFR 63.3090(c) or 40 CFR 63.3091(c), a record of the deviation to document the quarterly deviation report.

Each record shall be maintained for 5 years following the date of the occurrence, measurement, maintenance, corrective action, report, or record.  These records must be kept on-site for the first two years of this 5-year period of time.

[40 CFR 63.3130(c)(6)], [40 CFR 63.3131], and [40 CFR 63.3151(d)(1) and (e)]

2.	For any adhesive and/or sealer operation(s) where only compliant materials are applied during any month and the calculation for the mass average organic HAP content would not be required, the permittee shall maintain records to demonstrate that the organic HAP content of each material applied in the coating operation(s) is less than or equal to the applicable limitation contained in 40 CFR 63.3090(c) or 40 CFR 63.3091(c).  Each record shall be maintained for 5 years following the date of application of the coating or material.

[40 CFR 63.3152(c)] and [40 CFR 63.3131]

IV	Reporting Requirements

1.	The new source shall submit the initial notification of compliance status report no later than 60 days after the first full month following completion of all applicable performance tests, as required in this permit.  The initial notification shall include the following information:

a.	company name and address;

b.	statement by a responsible official certifying the truth, accuracy, and completeness of the content of the report (official’s name, title, and signature);

c.	the date of the report and beginning and ending dates of the reporting period;

d.	identification of the monthly compliance method for each adhesive and/or sealer coating operation, i.e., if using “compliant materials” or calculating the mass average organic HAP content each month;

e.	statement of whether the affected source achieved the emission limitations for the initial compliance period;

f.	if there was a deviation during the initial compliance period, a description of the deviation and statement of the cause and the calculations of mass average organic HAP content of the adhesives and/or sealers used to determine noncompliance with the applicable limits;

g.	all data and calculations and supporting documentation (information from supplier or manufacturer or summary of testing results if using only compliant materials) for the mass average organic HAP content of each adhesive or sealer applied for the month;

h.	all data and calculations used to determine the monthly mass average HAP content of all adhesive and sealer materials applied, other than materials used in glass bonding; and

i.	a statement of whether or not the work practice plan was developed and implemented.

[40 CFR 63.9(h)] and [40 CFR 63.3110(c)]

2.	The permittee shall submit semiannual compliance reports which shall be postmarked or delivered no later than July 31 and January 31 following the end of each semiannual reporting period.  The reporting period is each 6-month period of time ending on June 30 and December 31 of each year.  The semiannual compliance reports shall cover the previous 6 months of operation and shall contain the following information:

a.	company name and address;

b.	statement by a responsible official certifying the truth, accuracy, and completeness of the content of the report (signed with title);

c.	the date of the report and the beginning and ending dates of the reporting period;

d.	statement of whether the affected source achieved the emission limitations for the compliance period;

e.	if there were no deviations, a statement that there were no deviations from the emissions limitations during the reporting period; and

f.	deviations during the compliance period, which shall include the following information:

i.	the beginning and ending dates of each month during which the monthly average organic HAP content exceeded the applicable emission limitations of this subpart and permit;

ii.	the volume and organic HAP content of each material used that is subject to the applicable organic HAP content limit;

iii.	the calculation used to determine the average monthly organic HAP content for the month in which the deviation occurred; and

iv	the reason for the deviation.

g.	the calculation used to determine the monthly organic HAP emission rate, as required in this permit;

h.	if applicable, the calculation used to determine mass of organic HAP in waste materials;

i.	for each deviation from the work practice standards, a description of the deviation, the date and time period of the deviation, and the action taken to correct the deviation; and

j	a statement of the cause of each deviation;

[40 CFR 63.3120(a)]

3.	For any adhesive or sealer coating operation(s) that has been identified as only applying compliant materials, the permittee shall identify in the semiannual report any period of time where a material was applied that exceeded the organic HAP content limitation contained in 40 CFR 63.3090(c) or 40 CFR 63.3091(c).  The report shall document the date and duration of the exceedance, as well as the mass average organic HAP content calculation for the coating operations for the month during which the exceedance occurred.

[40 CFR 63.3152(b)]